Evaporated Coating Inc. Report

- Met with sale manager, one engineer.

- Inspection of mirror re-coating attempt:

- When preparing the surface, previous coating was coming off

- (conflicts with previous attempt to strip coating)

- Old coating removed, not very successfully. Problem with substrate?

- Re-coating was not successful. Surface is visually defective.

- Details on the technique used were asked but not given.

- Tour of facility denied.

- Chamber limited to 44’’ long

- Conclusion: ECI seems unable to re-coat the mirrors.

- Alternative: ECI gave me a 9’’ wide 36’’ long strip of lexan coated with Aluminum. We can try glue this on the mirror surface. If successful this could work since ECI can coat lexan very well.

- Inspection with the engineering of 2 LTCC elliptical mirrors, 1 hyperbolic, 4 cylindrical mirrors, Winston Cones.

- Left 2 WC for their analysis and re-coating test. They seem confident they can do it. Previous experience with HTCC positive.

- Left 2 cylindrical mirror to test their re-coating

- JLAB homework: test gluing lexan layer on mirror, check reflectivity and optical alignment.

- ECI homework: provide quote for lexan layers only, for WC and for cylindrical mirrors.

Temple University Report

- Met with Zein-Eddine

- Physics Lab Tour: He3 polarization, p-Terphenyl PMT coating

- Zein-Eddine very enthusiastic about this project. Evaporation tooling being refurbish, will be upgraded for 3 PMTs at once. Graduate students will be available to do the job

- Testing: Zein-Eddine have lamps to do wave-length dependant testing of ADC spectrum, including data acquisition.

- Plan is to test PMTs before/after the coating.

- Comparison with Quartz PMT will be done.

- JLAB homework: none for now.

- Temple U homework: refurbish the chamber.

Quantum Coating Report

- Met with three senior engineers, vice president of the company.

- Overview of LTCC project.

- Inspection with the engineering of 2 LTCC elliptical mirrors, 1 hyperbolic, 4 cylindrical mirrors, Winston Cones, spare mirror.

- Session with lots of QA, especially about the substrate.

- Engineers were perplexed on why the stripping didn’t work at ECI. While on the facility tour, one engineer tried to strip the spare mirror.

- Facility Tour:

- Dozen of chambers.

- Biggest chamber: 96’’. Used for the mirrors for the James Webb telescope.

- Production chain, including quality control, impressive

- Engineer was successful in stripping off coating of spare mirror.

- I left 1 hyp. Mirror, 2 cyl. Mirrors and one WC.

- Discussion about alternative strategy: coating lexan layers glued to mirror surface. This will be tested, worries about vaccum-safe of glue, and mirror substrate in general.

- JLAB homework: test gluing lexan layer on mirror, check reflectivity and optical alignment.

- QC homework: test stripping / re-coating on one mirror. Test re-coating of cyl. Mirrors.

- Overall impression positive.